Well...this program should show that certain people either

spread misinformation on purpose or just don`t know it better.

To measure the memory *BUS* performace and NOT the CPU

performance you have to understand how the cache works.

For CopyBack mode:

If you measure the WRITE bus performace you have to subtract

the LINE prefetch.

If you measure the READ bus performance you shouldn`t measure

the CPU's own cache controller latency as you would do with

something like that:

;gpr3=LINE start

lwz gpr4,0(gpr3)	;stall until this word is in the cache

lwz gpr5,4(gpr3)	;stall until this word is in the cache

lwz gpr6,8(gpr3)	;stall until this word is in the cache

lwz gpr7,12(gpr3)	;stall until this word is in the cache

lwz gpr8,16(gpr3)	;stall until this word is in the cache

lwz gpr9,20(gpr3)	;stall until this word is in the cache

lwz gpr10,24(gpr3)	;stall until this word is in the cache

lwz gpr11,28(gpr3)	;stall until this word is in the cache

..basicly you measure the LINE read + CPU Cache stalls.

Correct would be

lwz gpr4,0(gpr3)	;stall until this word is in the cache(hides line read)

lwz gpr5,32(gpr3)	;stall until this word is in the cache

;and that`s after the last line is read(hides line read)

lwz gpr6,64(gpr3)	;stall until this word is in the cache

;and that`s after the last line is read(hides line read)

lwz gpr7,96(gpr3)	;stall until this word is in the cache

;and that`s after the last line is read(hides line read)

and this in continues memory and with a greater loop. Then these

accesses are in pipeline so it doesn`t matter that the last

lwz gpr7 returns before the line is read complete.

For non COPYBACK modes the situation looks

different.

The Cache Speed test is disabled because it doesn`t work

reliable(yet).

P.S.

Source is provided to avoid another conspiracy thread.